ASSESSMENT OF VERY HIGH RESOLUTION SATELLITE IMAGES (VHR) FOR THE DEVELOPMENT OF TOPOGRAPHIC DATABASE

Zdzisław  Kurczyński

Wiesław  Wolniewicz

Institute of Photogrammetry and Cartography, Faculty of Geodesy and Cartography,

Warsaw University of Technology, Pl. Politechniki 1, Warsaw, Poland, 00-661

kurczynski@wp.pl, w.wolniewicz@gik.pw.edu.pl

Key words: VHR, orthorectification, orthophotomap, topographic data base, DTM, contents of VHR.

## Summary

In recent years Poland has carried out significant projects within the scope of Spatial Information Systems (SIP) on a scale hitherto unknown. These projects consist of the development of databases of a country range encompassing Digital Terrain Model, orthophotomaps and digital vector maps with the level of precision and detail that would satisfy the conditions for medium-scale maps (approx. 1:10,000).

In light of this situation, to speed up the works, is obvious the interest in high-resolution satellite images, and questions as to their usefulness in the realization of present and future tasks. This question is in time, because in Poland has just been established Regional Operation Centre (ROC) for acquisition and processing of Ikonos imagery.

Head Office of Geodesy and Cartography, in collaboration with Warsaw University of Technology, is preparing an research project which should answer the question if and to what degree such images can be useful in creating The Topographic Data Base (TBD).

For this purpose three test areas have been selected: urban, rural and mountainous. A very high resolution images from Ikonos, QuickBird and Eros systems ware purchased. The main goal of investigation are the orthorectification and creating of vector database. The results of carried works will enable to assess the usefulness of used imagery for creating of such database and compliance of possible extracted features with the standard of Topographic Data Base (TBD). Presented paper will consider the current progress in this area.

1. General background of the project

Today Poland is in the phase of substantial economic transition. In the field of geodesy and cartography there are the works in progress of the scale, scope and pace, which have not been undertaken ever before. It is connected, among others, with the process of making up some arrears we have in relation to the Western Europe as far as the coverage of our country with the modern geodesy and cartographic products in digital versions are concerned. We mean here, among others, such digital products as:

- Actual aerial photographs;

- Digital Terrain Model (DTM);

- Orthophotomap;

- Vector map functioning in GIS environment.

These products constitute the reference layers of the terrain information systems of different level of details and space range, which are being created nowadays. Possession of the said products constitutes a condition for the harmonized development, in particular in activities regarding the spatial arrangement of the state (planning in the fields of the land development, agriculture, natural environmental protection, combating the natural disasters, and others).

Preparation of the Land Parcel Identification System in the framework of IACS, constituting both the basis and condition for obtaining direct subsidies for agriculture in Poland from the European Union, has become substantial and urgent challenge.

In the framework of its statutory activities, the Head Office of Geodesy and Cartography (GUGiK) plays an initiating role in the field of production of the geodesy and cartographic products and their maintenance in updated status. In this field, the Office collaborates also with the Agency for Restructuring and Modernization of Agriculture, as well as with the Military Geography Division of Polish Armed Forces.

In order to satisfy the needs of the new and system solutions, the Head Office of Geodesy and Cartography established the new technical standards, manuals and other papers standardizing technological processes.

One of the more ambitious and urgent needs is a creation topographic data base of a level of details and accuracy more or less similar to traditional topographic map scaled 1:10 000. Several years ago, the Head Office of Geodesy and Cartography undertook the works in this field in form of the pilot projects. In result of these activities, one has elaborated the new standard described in the “Technical Guidelines Topographic Data Base (TBD)”.

The Topographic Data Base is composed of the following main elements:

- Base of the Digital Terrain Model in sectional division of height accuracy 1 meter,

- Base of the orthophotomaps in sectional division with terrain pixel 0.5 meter and planar accuracy 1.5 meter (i.e. 3 pixels),

- Spatially „continuous” vector topographic data base of accuracy and contents identical with the traditional topographic map scaled 1:10 000.

The basic source materials for preparation of the above-mentioned components are the aerial photographs and their photogrammetric processing. Manufacturing of the products of quality meeting the “Technical Guidelines” requires the scale of photographs from the range from 1:20 000 to 1:30 000. The national standard provides for taking the photographs in scale 1:26 000. However, the “Technical Guidelines” allow for use of other source data, such as: high resolution satellite images, photographs taken with a use of aerial digital camera, and laser scanning.

The implementation works on creation of TBD data base are in progress. One should bear in mind however that this is a very enormous undertaking: the entire country is covered by over 17,000 TBD modules (TBD module is an equivalent of a sheet of topographic map scaled 1:10 000). This products is created with a use of absolutely state-of-the-art technology, but the pace of its development is unsatisfactory, and the costs are just enormous. Creation of the vector  data base in TBD standard constitutes particularly expensive and time consuming task. It is caused, among others, by the very enhanced contents of this data base. At the same time the products of TBD parameters is urgently needed, and for many institutional clients such a product would meet their expectations only if it can cover the entire country, or at least its key areas, within relatively short period of time, some 4-5 years. For many customers the time priority prevails over the richness of the details included in the vector map. In connection with that one considers the optional creation of TBD vector map of limited contents. However, it must be highlighted that we mean here only the reduced contents of the map with its positional accuracy still maintained. Such a product would be created on areas of the weaker urban development, and in need it would be supplemented and updated in the future.

In the meantime, the VHRS (Very High Resolution Satellites) were created and put into operational phase. In this situation, one asked a natural and obvious question about usefulness of the very high resolution satellite images for the creation of components in the TBD data base, and in particular in vector data bases. This very question constitutes the background for the foundation of a special project entitled: „Elaboration of vector elements of the topographic data base as well as discrete technology of multi-spectral analysis of surface changes on the basis of the very high resolution satellite images” which has been ordered by the Head Office of Geodesy and Cartography, and co-financed by the Head Office of Geodesy and Cartography and the Committee of Scientific Research of Poland, which is executed by the Institute of Photogrammetry and Cartography of the Warsaw Technical University. The above presented issues constitute one of the two directions of research undertaken in this very Project.

Within the scope of the task connected with acquiring of vector data for TBD topographic data bases, the goal of the Project is to answer a question on the scope of usefulness of the very high resolution satellite images for establishment and updating of TBD vector data base in various technical, organizational and economic conditions. Reliability of such an answer should be checked upon and proved in the production conditions. It has also been presumed that in result of this Project one will formulate recommendations for the Head Office of Geodesy and Cartography regarding the direction of modernization of TBD standard for creation of an option to use VHR for the foundation of TBD. The indirect result will be the recommendations in the fields of orthorectification technology of the very high resolution satellite images and the selection of tool platform for realization of this task.

Implementation will constitute an important part of this Project. In order to obtain the results of high reliability, the stage of experimental elaboration of the vector elements of TBD should take place in manufacturing environment within the selected production enterprise.

2. Testing fields and satellite images

It is recommended to undertake evaluation of an opportunity for creation of TBD vector data bases based upon very high resolution satellite images by means of vectoring the orthophotomaps made of these images. The orthophotomaps should have the following characteristics:

- Sufficient geometrical accuracy,

- Sufficient potential for interpretation enabling for the recognition and measurement of the objects constituting the contents of the data base.

These two requirements force the conduct of research and experimental works in two directions:

- Evaluation of the potential for orthorectification of satellite images in terms of accuracy of generation of the satellite orthophotomaps.

- Evaluation of the potential for creation of TBD vector data bases based upon the satellite orthophotomaps.

In experimental phase, the above-mentioned two directions should be referred to areas, which are representative for the country, so to the following ones:

- Areas of intensive urban development,

- Rural areas,

- Flat areas as well as the areas of a complex ground relief and considerable de-leveling.

Additional criterion for selection of testing fields was a usefulness of the satellite images for the second task in the research Project connected with a recognition of surface changes and establishment of automatic controlling mechanisms (in the context of supervision of the cultivation and crops for the needs of IACS). Taking into consideration the above-mentioned goals, one has selected three typical test objects:

- Testing object „Warsaw”,	
- flat terrain, area of intensive urban development.

- Testing object „Nowy Targ”,	
- terrain of considerable de-leveling (up to 600 meters within a single scene), of rich ground relief and rich with topographic contents.

- Testing object „Włocławek”,	
- flat terrain, open area, of a definite domination of agricultural usage.

Warszawa

Nowy Targ

Włocławek

The objects selected in this way are representative – in terms of the ground relief and land coverage – for the majority of the surface of our country (fig. 1).

## Figure 1: Localization of the testing objects

For the areas of the testing objects one purchased the very high resolution satellite images from the following available systems:

- EROS A 1 (Image Sat Int.),

- Ikonos – 2 (Space Imaging),

- Quick Bird (Digital Globe).

In the event of  EROS system, these are the panchromatic images with the terrain pixel 1.8 meter. In the event of Ikonos system, these are the panchromatic images with the terrain pixel 1.0 meter, and for the “Włocławek” object additionally the multi-spectral image with the terrain pixel 4.0 meters, obtained in spectrums: blue, green, red and infrared. In the event of Quick Bird system, these are the panchromatic images with the terrain pixel 0.6 meters, and for the “Warsaw” object additionally the multi-spectral image with the terrain pixel 2.4 meters.

The images were ordered in the possibly most raw and original (unprocessed) format and form enabling for their further elaboration. This means for Ikonos system, a type of product Geo Ortho Kit, and for the QuickBird system, a product Basic 1B. These products include the elements of orientation registered by the satellite devices, and as such they are the most useful for further survey processing, including generation of the orthophotomaps.

3. Orthorectification of satellite scenes – preliminary data

Digital Terrain Model

Exact orthorectification of  satellite images requires information on configuration of the surface of a given ground in form of DTM. Accuracy of the orthoimage generation depends on the DTM quality (height accuracy). Requirements in relation to DTM depend on the expected accuracy of orthorectification and on the deflection angle of the camera optical system from nadir. The greater the deflection is, the stronger influence of DTM errors upon accuracy of orthorectification is, and therefore the greater are the requirements for DTM accuracy. For these reasons for survey works one uses the images obtained in the optical camera deflection from nadir up to 15º – 20º. Geometrical analysis proves that for such images, in order to obtain accuracy of orthorectification on the level of 1.0 – 1.5 meter, it is sufficient when DTM has height accuracy of some 5 meters.

| Type of DTM | Grid size | Warsaw |  | Nowy Targ |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Number of GCPs | RMSE [m] | Number of GCPs | RMSE [m] |
| DTED 0 | 30´´ x 30´´ (app. 900 m x 900 m) | 39 | 10.56 | - | - |
| DTED 1X | 3´´ x 3´´ (app. 90 m x 90 m) | 39 | 2.41 | 101 | 5.54 |
| DTED 2 | 1´´ x 1´´ (app. 30 m x 30 m) | 39 | 0.66 | 101 | 3.87 |
| DTED 2X | 10 m x 10 m | - | - | 101 | 4.21 |

Table 1: Parameters of Digital Terrain Models used for orthorectification

a)			b)			   c)			     d)

Figure 2: Comparison of used DTMs: a) DTED Level 0, b) DTED Level 1X, 
c) DTED Level 2, d) DTED Level 2X

For the reason of practical investigation of influence of DTM quality upon the process of orthorectification, one used several DTMs of different standards:

- DTED Level 0	
DTM of global range, produced and made available free of any charge by NIMA (National Imagery and Mapping Agency). It is DTM in GRID structure of the mesh 30´´ x 30´´ (some 900 meters x 900 meters).

- DTED Level 1X	
DTM of global range, prepared on the basis of radar interferometry during the mission of a space shuttle SRTM (Shuttle Radar Topography Mission). It has the GRID structure of a mesh  3´´ x 3´´ (some 90 meters x 90 meters) and 1´´ x 1´´ (some 30 meters x 30 meters). For experimental works one used DTM of the mesh 3´´ x 3´´.

- DTED Level 2	
DTM covering the entire area of Poland. It has been prepared on the basis of a relief obtained from topographic maps scaled 1:50 000. It has the GRID structure of the mesh 1´´ x 1´´ (some 30 meters x 30 meters), which is also available in the metric structure of GRID of the mesh 25 meters x 25 meters.

- DTED Level 2X	
DTM prepared on the basis of a relief obtained from topographic maps scaled 1:25 000. It has the GRID structure of the mesh 10 meters x 10 meters.

DTM used for experimental works were evaluated in terms of accuracy on the area of the testing objects „Warsaw” and „Nowy Targ” on the basis of the GCP points measured with the GPS technology. Summarized results of this analysis are illustrated in Table 1 and Figure 2.

The experiments conducted with different DTMs showed that even for the strongly undulating terrain (for instance the „Nowy Targ” object), DTED Level 2 provided the sufficient accuracy for orthorectification.

## GCP points

Both, the analysis and the experiments conducted have proved that the very prime factor determining accuracy of orthorectification is the number, localization and quality of the GCP points used in the process of geometrical correction (GCP – Ground Control Points).

Evaluation of the process of orthorectification should not however be provided on the basis of residual deviations on the GCP points obtained in the adjustment process, that is such evaluation should not be based upon internal measures of accuracy obtained in the process of adjustment. It results from specifics and behavior of the polynomial methods of approximation. Reliable evaluation of accuracy of the orthorectification process is conducted on the basis of deviations in the check points that is in the points of the measured localization in terrain, which are not involved in the adjustment process (they are treated as the determined points), and which serve for evaluation of accuracy of the orthorectification process.

In the conducted experimental works, on each image one defined a considerable number of points, substantially transgressing the needs of the orthorectification as such (amounting to several dozens per scene). These were the natural points (i.e. the terrain details). In the adjustment process, some of these points were treated as the GCP points, while their remaining part – as the Independent Check Points - ICP. But proper selection of the points, their terrain survey and identification on the images constituted the key for success.

Designing the deployment of the GCP points on the scene, one attempted to provide a maximum uniform distribution of these points on entire area. It constitutes of the conditions for achieving possibly equal accuracy of results of orthorectification for entire imaging. Each localization situated on the rim of scene was chosen in such a way to limit to maximum the areas situated outside the area covered by the points. Such areas are particularly dangerous due to extrapolation of results from adjustment of the control network observed within them and possibility for emergence of areas with considerably worsen results.

When projecting the GCP points, one tried to select such terrain details, which would enable for their identification on the image with the sub-pixel accuracy. For the terrain survey one selected a measurement with GPS technology GPS FastStatic. This method enabled for obtaining the coordinates of all the GCP points with the identical and high accuracy within relatively short period of time. Additional advantage of this method is its relatively low labor-intensive character necessary for obtaining data with the required precision.

The surveys were executed with a use of two satellite receivers Trimble 4700 with the micro-centered antennas. On each localization one selected two of previously designed points for measurement. The antennas were positioned on the tripods above the GCP points.

4. Process of  orthorectification and analysis of the results

All the 8 satellite images obtained from the satellite systems were subject to orthorectification. They covered the three following testing objects:

- Testing object „Warsaw” – images EROS, Ikonos and QuickBird,

- Testing object „Nowy Targ” - images EROS, Ikonos and QuickBird,

- Testing object „Włocławek” - images EROS and Ikonos.

On each and every of the images one measured several dozens of the points with the application of the above described method.

Geometrical adjustment was conducted with a use of two basic methods:

- The method based upon the parametrical model (rigorous method),

- The method based upon rational polynomial model (RPC).

In experimental works, one used two software packages, which are primary in the field of orthorectification of satellite images:

- PCI Geomatica 9.0, enabling for the use of both correction methods,

- Erdas Imaging 8.5, enabling for the use of the rational polynomial method (RPC).

One realized a lot of correction variants for each image with analysis of the results obtained. Quality of correction was evaluated on the basis of residual errors, which were always determined on a considerable number of check points (ICP) and by analysis of a character of residual errors within the scene (distribution of residual vectors of distortions).

The research in the field of orthorectification was conducted in the following areas:

- Evaluation of influence of DTM quality upon the results of geometrical correction,

- Evaluation of the number and distribution of the GCP points used upon the results of geometrical correction,

- Evaluation of influence of correction method (parametrical method and RPC polynomial method with a use of parameters of the models supplied with the image in form of metadata, or without such data),

- Evaluation of the usefulness of software.

This report presents only some examples of the results obtained. The following tables 2 – 5 and on drawings 3 – 6 presents the results of correction of images from Ikonos and QuickBird from testing fields „Warsaw” and „Nowy Targ” obtained from the software PCI Geomatica. The tables present the average errors and maximum errors of correction separately for each coordinate X and Y, with a use of different digital terrain models (DTM) and correction methods. The errors were evaluated on the check points (ICP) after adjustment with a use of 9 GCP points. Moreover, the schemes illustrate accuracy of correlation in form of the average location errors (i.e. a resultants of errors X and Y) in the function of the GCP points used for such correction, as well as the applied DTM. Analysis of the results obtained enables for formulation of the following general conclusions:

- For flat area (testing object „Warsaw”) with a use of digital terrain DTED Level 1X and 2, RPC method enables for correction of Ikonos image on the level of accuracy 1.0 – 1.3 meters almost independently from the number of GCP points (Fig. 3).	
For DTED Level 0 the results are worse by 0.5 – 1.0 meter. 	
Parametrical method requires a use of minimum 8 GCP points in order to achieve similar accuracy.

- For exactly the same conditions, the QuickBird image can be corrected with accuracy some 2.0 meters with a use of 2 GCP points. The use of 8 GCP points and parametrical method results in accuracy of 1.3 meter. Impact of DTM is similar for both methods of correction (Fig. 4).

- For the foot-hills area (testing object „Nowy Targ”), RPC method enables for correction of Ikonos image with accuracy some 1.5 meters almost independently from the used DTM and the number of GCP points (Fig. 5).	
Use of parametrical method and minimum 10 GCP points produces similar result.

- For the QuickBird image and RPC method one obtains a correction with error slightly exceeding 3.0 meters already with a use of 2 GCP points. Parametrical method with 9 points enables for obtaining two times less errors of the QuickBird image correction (Fig. 6).

- Digital Terrain Model DTED Level 1X coming from SRTM is sufficient for orthorectification of the very high resolution satellite images.

- Even for extremely undulating terrain, the Digital Terrain Models DTED Level 2 and DTED Level 1X (SRTM) produce similar results for both images and correction methods.

| DTM | Parametric approach |  |  |  | RPC approach |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | RMS [m] |  | Maximum [m] |  | RMS [m] |  | Maximum [m] |  |
|  | X | Y | X | Y | X | Y | X | Y |
| DTED 0 | 0.78 | 1.2 | 1.32 | 2.16 | 0.65 | 1.18 | 1.36 | 3.52 |
| DTED 1X | 0.65 | 0.8 | 1.56 | 1.92 | 0.55 | 0.93 | 1.08 | 2.31 |
| DTED 2 | 0.65 | 0.86 | 1.41 | 1.75 | 0.54 | 0.88 | 1.13 | 2.30 |

Table 2: Accuracy of Ikonos image correction with 9 GCP, testing area „Warsaw” (RMS and maximum errors on the ICP points) for parametrical and polynomial (RPC) methods, and for different DTM

a)						b)

Figure 3: Accuracy of Ikonos image correction, testing area „Warsaw”, 
a – parametrical method, b – polynomial method (RPC)

| DTM | Parametric approach |  |  |  | RPC approach |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | RMS [m] |  | Maximum [m] |  | RMS [m] |  | Maximum [m] |  |
|  | X | Y | X | Y | X | Y | X | Y |
| DTED 0 | 1.03 | 0.66 | 2.21 | 1.30 | 1.10 | 1.14 | 3.37 | 2.16 |
| DTED 1X | 0.88 | 0.60 | 2.32 | 1.42 | 1.29 | 1.07 | 4.02 | 2.06 |
| DTED 2 | 0.92 | 0.62 | 2.35 | 1.42 | 1.33 | 1.05 | 4.04 | 2.03 |

Table 3: Accuracy of QuickBird image correction with 9 GCP, testing area „Warsaw” (RMS and maximum errors on the ICP points) for parametrical and polynomial (RPC) methods, and for different DTM

a)						b)

Figure 4: Accuracy of QuickBird image correction, testing area „Warsaw”, 
a – parametrical method, b – polynomial method (RPC)

| DTM | Parametric approach |  |  |  | RPC approach |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | RMS [m] |  | Maximum [m] |  | RMS [m] |  | Maximum [m] |  |
|  | X | Y | X | Y | X | Y | X | Y |
| DTED 1X | 0.88 | 1.01 | 2.12 | 2.85 | 1.03 | 1.27 | 2.84 | 3.15 |
| DTED 2 | 0.75 | 0.93 | 1.42 | 2.50 | 0.96 | 0.83 | 2.64 | 2.50 |
| DTED 2X | 0.88 | 1.10 | 1.60 | 2.47 | 1.08 | 1.21 | 2.91 | 2.97 |

Table 4: Accuracy of Ikonos image correction, testing area „Nowy Targ” (RMS and maximum errors on the ICP points) for parametrical method (with13 GCP) and polynomial (RPC) method (with 9 GCP), and for different DTM

a)						b)

Figure 5: Accuracy of Ikonos image correction, testing area „Nowy Targ”, 
a – parametrical method, b – polynomial method (RPC)

| DTM | Parametric approach |  |  |  | RPC approach |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | RMS [m] |  | Maximum [m] |  | RMS [m] |  | Maximum [m] |  |
|  | X | Y | X | Y | X | Y | X | Y |
| DTED 1X | 1.31 | 1.21 | 3.16 | 3.57 | 2.93 | 1.57 | 6.28 | 3.06 |
| DTED 2 | 1.00 | 0.94 | 3.14 | 2.60 | 2.72 | 1.58 | 5.63 | 2.85 |
| DTED 2X | 1.18 | 0.93 | 3.14 | 2.87 | 2.79 | 1.57 | 6.19 | 2.92 |

Table 5: Accuracy of QuickBird image correction with 9 GCP, testing area „Nowy Targ” (RMS and maximum errors on the ICP points) for parametrical and polynomial (RPC) methods, and for different DTM

a)						b)

Figure 6: Accuracy of QuickBird image correction, testing area „Nowy Targ”, 
a – parametrical method, b – polynomial method (RPC)

Figure 7 illustrates another example of adjustment. It is a result of Ikonos image correction from the „Warsaw” testing area, executed with a use of DTM DTED Level 2 and both correction methods, realized in PCI Geomatica and Erdas Imaging environment.

One applied various correction variants. In PCI Geomatica environment one used as follows:

- Parametrical method  – PCI 1,

- Polynomial method (RPC) with the coefficients supplied by the distributor – PCI 2,

- Polynomial method (RPC) with the calculated coefficients (4 coefficients) – PCI 3,

In Erdas Imaging environment one used as follows:

- Polynomial method (RPC) with the 0 degree polynomial – ERDAS 0,

- Polynomial method (RPC) with the 1st degree polynomial – ERDAS 1,

- Polynomial method (RPC) with the 2nd degree polynomial – ERDAS 2.

The courses of the residual after correction errors diagrams for different methods and various number of the GCP points confirm the capacity for adjustment of Ikonos images on the level of 1.0 – 1.2 meters, i.e. slightly above a single pixel of image already with a use of 8 – 9 GCP points.

Figure 7: Comparative analysis of the results of Ikonos image correction – object „Warsaw”. Program PCI Geomatica – parametrical method – PCI 1, polynomial method with the coefficients from distributor – PCI 2, polynomial method with the calculated coefficients (4 coefficients) – PCI 3, Program Erdas Imaging – polynomial method with the correction of coefficients by the 0 degree polynomial – ERDAS 0, the 1st degree polynomial – ERDAS 1, the 2nd degree polynomial – ERDAS 2

Example results of EROS image correction (parametrical method, software PCI Geomatica, testing area „Nowy Targ”) are illustrated on Figure 8.

Applying parametrical method for correction of the image type EROS for difficult area (large de-leveling), one obtained the average positioning error within a range of single pixel (some 2.0 meters) already with the 10 GCP points.

Fig. 8.  Accuracy of EROS image correction, testing area „Nowy Targ” (average error of position in the function of the GCP points number) – parametrical method.

5. Generation of the digital orthophotomaps

All investigated images were subject to the process of geometrical correction, and then one generated the digital orthophotomaps from them, with the terrain pixel respectively:

- For EROS source image – orthophotomap with the terrain pixel 1.8 m,

- For Ikonos source image – orthophotomap with the terrain pixel 1.0 m,

- For QuickBird source image – orthophotomap with the terrain pixel 0.6 m.

In the process of orthorectification for creation of orthophotomap one used the bilinear method of re-sampling. The resulting orthophotomaps were saved in GeoTIFF format.

Images from Ikonos system for testing objects „Warsaw” and „Włocławek” were in the form of products including both the panchromatic images (black-and-white), as well as the multi-spectral ones. From the multi-spectral images one can create colorful compositions in the real colors (by combination of spectral bands: blue, green and red), or in the false colors (by combination of spectral bands: green, red and infrared).

In the framework of experimental works, one additionally created the colorful pan-sharpened composition. It consists in a fusion of the panchromatic very high resolution image (with the terrain pixel 1 meter) with the multi-spectral image (with the terrain pixel 4 meters). In result of special processing, one obtains a colorful composition (in the real or false colors), but with the pixel as in the case of panchromatic image, i.e. 1 meter.

In order to obtain such product, one applied the following series of transformations:

- Orthorectification of panchromatic image on the basis of the GCP points,

- Creation of the digital, black-and-white orthophotomap,

- Orthorectification of multi-spectral image in relation to the created digital, black-and-white orthophotomap in the “image to image” mode,

- Integration of the digital, black-and-white and color orthophotomaps and creation of the colorful orthophotomap with 1-meter pixel (the pan-sharpened orthophotomap).	
Such integration was executed with the Erdas Imaging software with the application of the Principal Component method, while for re-sampling of the multi-spectral image one applied the bilinear method.

6. Evaluation of the capacity for creation of the vector data of topographic data base (TBD) on the basis of the very high resolution satellite images

Evaluation of the cartographic potential of the very high resolution satellite images should take into consideration the following two aspects:

- Geometrical aspect, in order to answer the question if a product created on the basis of the satellite images meets the requirements of positioning accuracy,

- Interpretation aspect, in order to answer the question on the scope of possible extraction of the terrain object and their attributes as compared with the contents of created digital map, for example in standard TBD.

Ikonos satellite images provide for generation of the orthophotomaps with the terrain pixel 1 meter, while the QuickBird images – with the pixel 0.7 m. Such orthophotomaps have positional accuracy on the level of 1.0-1.5 meter. Despite the fact that such orthophotomaps can be plotted even in scale 1:5 000, in terms of required pixel they still do not meet the requirements of orthophotomap in TBD standard, which requires the pixel 0.5 meter. However, the orthophotomaps produced on the basis of satellite images can meet the requirements of positional accuracy.

Response to the question regarding the contents of satellite images is far more difficult and ambiguous. Comparison of the required resolution of digital images for creation of the topographic vector map and orthophotomap confirms a well-known fact that for creation of the digital orthophotomap it is sufficient to have images of lower resolution capacity than for creation of vector map of the same scale (for instance: photos 
1:26 000 enable for the creation of orthophotomap scaled 1:5 000, but only for creation of the vector map scaled 1:10 000).

Comparing both criteria defining the cartographic potential of images, i.e. geometrical accuracy of elaboration and interpretation capacity, one can easily notice that available satellite images can more easily meet the requirements of geometrical accuracy that the requirements as to their contents. Critical factor limiting the cartographic potential of available satellite images is their limited contents, while only in the second instance – their geometrical accuracy.

In the literature we can meet different opinions on the scope of usefulness of the very high resolution satellite images [1,2,4]. The opinions on cartographic potential of these images are ambiguous. There may be several reasons for existing ambiguity:

- Lack of sufficient experimental results conforming practical scope of usefulness of images, in particular in the context of cartographic works conducted in larger scale. We have very little information on any use in larger scale of the very high resolution satellite images for the „routine” cartographic works conducted by the governmental cartographic institutions – the counterparts of Polish Head Office of Geodesy and Cartography. In actuality, we have no such data what may be explained by the fact that we are in the stage of establishment of the market for the very high resolution satellite images, and their use is in the phase of experiments and implementations.

- When commenting on the cartographic potential of images, the authors do not always distinguish between their survey potential and interpretation potential. Quite often, they adopt only the measurement accuracy as the criterion of usefulness, while the factor limiting the usefulness of satellite images are not their high survey potential, but their limited interpretation capacity.

- A reason for misunderstanding may also be ambiguous comprehension of resolution strength of digital images. Quite often, the terrain dimension of pixel is wrongly taken as the power resolution of the image.

- The scientific circles are flooded by information disclosed in the advertisements. For obvious reasons, in such messages one exaggerates about advantages of satellite images, or present certain problems from one point of view only.

- Opinions about the potential of satellite images are formed on the basis of selected samples of such images of very high resolution supplied by the distributors, which were obtained in very good atmospheric conditions. However, the real quality of these images may be very much diversified, chiefly due to impact of the atmosphere. One should take into consideration that the atmosphere has its degrading influence upon the quality, especially on such areas as Europe.

- The authors comment on the usefulness of satellite images for creation of the cartographic products of certain scale, but they take into consideration only a creation of the orthophotomaps, while they should rather distinguish between the requirements for creation of the orthophotomaps and digital topographic maps.

Evaluation of the contents of images in the context of creation and updating the maps in a given scale and of the pre-defined scope of the contents may depend on the following:

- Quality of image, particularly the one influenced by the atmosphere,

- Time of day and year of imaging (influence of the position of the sun and the vegetation status masking the terrain details),

- Land characteristics (urban and developed, rural area),

- Different types of contents that obligatory are present on the topographic maps in various countries,

- Technology of elaboration (stereo-digitalization or mono-plotting),

- The task assumed: creation of the new map or update of existing one,

- Other conditions.

Cartographic potential of images may be expressed by reference to aerial photographs of similar potential. In the field of aerial photographs, the years of practical experience have established technological relations between the scale of map and the scale of aerial photographs of which such map may be created. When evaluating the satellite images potential, one also tries to estimate these potential by giving an equivalent of the scale of aerial photographs of a comparable scope of contents. It is by no means an easy task, and there are quite ambiguous opinions in this matter in the environment of the photogrammetry experts. Experience of the authors enables them for producing the following opinion: cartographic potential (in terms of measurement and interpretation) of the very high resolution satellite images type Ikonos or QuickBird with a pixel close to 1 meter constitutes an equivalent of the aerial photographs of the scale 1:25 000 – 1:40 000.

The above-mentioned statement is very important, both from technical and economic points of view. It shows what kind of aerial photographs may be replaced by the very high resolution satellite images. One should note that in the literature you can meet more “optimistic” evaluation of the satellite images potential.

When evaluating the opportunity for TBD creation and updating based upon the very high resolution satellite images, one should take into account – besides technical criteria – also the economic ones. In actual state, the prices of satellite images are 1.5 – 2.0 times more expensive that the prices of aerial photographs of comparable potential. Today, also the creation of orthophotomap is some 2 times more expensive. However, it is worthwhile noticing that the market for the very high resolution images is just developing. The next systems are expected to come, so we can foresee that in the future their prices will fall.

Creation of the orthophotomap from satellite images is a relatively simple, fast and cheap process (excluding the cost of the images). This process is even simpler than the preparation based upon aerial photographs. So, we can expect that the domestic production of such maps will trigger and develop. In the middle of 2004, one launched in Poland the Satellite Center for Regional Operations (SCRO) dealing with receiving and processing of Ikonos images. This is the third center of this type in Europe, and this fact constitutes an important circumstance of the matter.

The research is progress is to show the degree of usefulness of the Very High Resolution Satellite Images for creation of vector data bases in TBD standard, i.e. the vector data bases of positioning accuracy and details equal more or less the traditional topographic map scaled 1:10 000. Pursuant to the binding standard, which has been specified for in the Technical Requirements, the main source of data for vector data bases is the orthophotomap with the pixel 0.5 meter produced from aerial photographs in scale 1:26 000. The vector data base per se is created by screen digitalization of such orthophotomap and organization of data into proper structures pursuant to TBD standard.

Evaluation of the usefulness of satellite images for this task is provided in the attempts in form of independent elaboration on the basis of digital orthophotomaps from different satellite systems for individual testing objects. Such elaboration consists in gathering and organizing into proper structures the vector data bases for the needs of TBD, conducted in accordance with guidelines based upon previously created digital satellite orthophotomaps. This means that the technological process of creation of individual elaboration works is as close as possible to the actual process of TBD production with such a difference that the source material for acquiring data are the digital satellite orthophotomaps only.

In order to obtain reliable results, which are important and necessary for the production, it is desirable to conduct the process of vectoring and acquiring attributes of the objects in conditions similar to maximum to the conditions of the real production, with a participation of experienced operators of cartographic stations, who previously executed in the framework of their daily routine the TBD elaboration works from aerial photographs.

The works obtained in this way in ARC/INFO environment are then subject to quantitative and qualitative analyses comprising each individual elaboration for particular testing areas. This analysis is to evaluate the degree of possible extraction of the objects and their attributes for individual elaboration works (different orthophotomaps and various areas) as per particular class of the objects, pursuant to systematization adopted in the TBD Technical Requirements. The elaboration works are compared between each other as well as with the referential works, i.e.:

- With existing data base elaboration works in TBD standard,

- With the ones obtained on the basis of the orthophotomaps from aerial photographs,

- With existing topographic maps scaled 1:10 000,

- With the terrain survey (on a limited area).

In the framework of individual testing objects, exactly the same land area is subject to elaboration, but on the basis of different source data.

Up to now, one has executed the attempt of vectoring of created orthophotomaps and elaboration of vector maps. However, these were laboratory trials realized by the University scientists. This does not offer any grounds for achievement of reliable factual evaluation of usefulness of the Very High Resolution Satellite Images for the specified task, with such reliability that would enable for the formulation of conclusions suitable for the production practice. For this reason, the works on the vectoring the orthophotomaps produced are simultaneously conducted in the production conditions in one of the enterprises dealing with the manufacturing of TBD.

The results obtained up to now let us state that the scope of contents of the orthophotomaps generated from images type Ikonos or QuickBird is narrower than orthophotomaps with the pixel 0.5 meter generated from aerial photographs scaled 1:26 000. This means that the vector data base in TBD standard can be achieved, but with incomplete scope of its contents. One could expect such a result; however, this result will have practical importance, because it will enable for the definition of the new standard of vector map with the limited scope of contents but maintaining at the same time the same positioning accuracy, which is now required by TBD. This in turn will enable to offer the countrywide coverage with such a product rather soon.

7. Conclusions

- The analysis and the experiments conducted have proved that the very prime factor determining accuracy of orthorectification of VHR images is the number, localization and quality of the Ground Control Points (GCP) used in the process of geometrical correction.

- Digital Terrain Model coming from SRTM is sufficient for orthorectification of the VHR.

- The VHR images type Ikonos or QuickBird can be corrected with accuracy of 1.0–1.5 m by applying parametrical method and 8-10 GCPs. Polynomial method (RPC) needs as few as 2 GCPs.

- Evaluation of the cartographic potential of VHR images should take into consideration both, geometrical and interpretation aspect. Critical factor limiting the cartographic potential of available satellite images is their limited contents, while only in the second instance – their geometrical accuracy.

- Cartographic potential (in terms of measurement and interpretation) of the VHR images type Ikonos or QuickBird with a pixel close to 1 meter constitutes an equivalent of the aerial photographs of the scale 
1:25 000 – 1:40 000.

- When evaluating the opportunity of VHR for the development of topographic data base, one should take into account – besides technical criteria – also the economic ones. In actual state, the prices of satellite images are 1.5–2.0 times more expensive that the prices of aerial photographs of comparable potential. Today, also the creation of orthophotomap is more expensive. However, it is worthwhile noticing that the market for the very high resolution images is just developing. The next systems are expected to come, so we can foresee that in the future their prices will fall.

References

[1]. Jacobsen K., Passini R., 2003 Accuracy Of Digital Orthophotos From High Resolution Space Imagery, Proceedings of the Workshop High Resolution Mapping from Space 2003, Hanover 2003.

[2]. Jacobsen K., Generation Of Orthophotos With Carterra GEO Images Without Orientation Information

[3]. Kurczynski Z. Wojtynek R., Wolnieiwcz W., 2004. Assessment of VHR Images Utilization for the Needs of the Polish National Geodesy Authorities, GIS 2004 , Croatia.

[4]. Nale D. K., 2002. QuickBird - Aerial Product Comparison Report (prepared by EMAP International for DigitalGlobe). www.digitalglobe.com.

[5]. Wolniewicz W., 2004. “Assessment of VHRS images “ – invited paper Eurimage Meeting , Rome.

[6]. Wolniewicz W., 2004. “Assessment of Geometric Accuracy of VHR Satellite Images”, XX Congress ISPRS, Istanbul.

[7]. Wolniewicz W., Jaszczak P., 2004. “Orthorectification of Very High Resolution Satellite Images”, XXV ACRS, Thailand.